LEV log book

# System details

Use one log book for each LEV system. Keep it with the system's user manual, its commissioning report and its latest thorough examination and test report.

| Company |  |
| --- | --- |
| Site address |  |
| System name or ID number |  |
| Where the system is (building, room or area) |  |
| Process and substances it controls |  |
| Hoods (how many, and what type) |  |
| Designed and installed by |  |
| Commissioning date and report reference |  |
| Where the user manual is kept |  |
| Person responsible for the system |  |
| Months between thorough examinations and tests (14 at most) |  |
| Date of the last thorough examination and test |  |
| Next thorough examination and test due |  |

Source: HSE leaflet INDG408.

LEV log book

# Check schedule

Set out what to check on each part of the system, how often, and who checks it. Copy the checks and how often they are due from the system's user manual. The filled rows are HSE's examples.

| Part of the system | What to check | How often | Who checks |
| --- | --- | --- | --- |
| Hoods | Airflow indicator reading, physical damage, blockages, clutter in the way |  |  |
| Ducts | Damage, wear and partial blockage, especially flexible ducts |  |  |
| Dampers | Still in the set position |  |  |
| Air cleaner or filter | Damage, pressure reading across the filter, failure alarms |  |  |
| Fan | Changes in noise or vibration, power use, clean impeller |  |  |
| Operators | Using the system correctly, for example hood position and the work kept inside the capture zone |  |  |

Source: HSE guidance HSG258, paragraphs 315, 319 and 320.

LEV log book

# Daily check by the person using it

Answer each question before use. Write Y for yes or N for no. If question 4 or 5 is a yes, or question 1, 2, 3 or 6 is a no, tell your supervisor straight away.

Week starting: ________________________________________________

System name or ID: ________________________________________________

| Question | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Does the indicator show the LEV is working properly? |  |  |  |  |  |  |  |
| 2. Is it taking away all the harmful dust, mist, fumes and gas? Remember, some of these may be invisible. |  |  |  |  |  |  |  |
| 3. Are you close enough to it so it can do its job properly? |  |  |  |  |  |  |  |
| 4. Are there any signs it is not working properly, like smells or settled dust? |  |  |  |  |  |  |  |
| 5. Are there any unusual noises or vibration coming from it? |  |  |  |  |  |  |  |
| 6. Has it been thoroughly tested, with a 'tested' label that is within date? |  |  |  |  |  |  |  |
| 7. Have you told your supervisor about anything you think may be wrong? |  |  |  |  |  |  |  |
| Initials |  |  |  |  |  |  |  |

Source: Questions from HSE pocket card INDG409, Time to clear the air!.

Contains public sector information published by the Health and Safety Executive and licensed under the Open Government Licence.

LEV log book

# Weekly check

HSE's COSHH code of practice and guidance says LEV should be visually checked at least once a week. Write OK, or describe what you found.

| Date | Hoods | Airflow indicator reading | Ducts | Dampers | Filter pressure reading | Fan noise and vibration | Used correctly | Faults found and action taken | Name | Signature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Source: HSE L5, paragraph 172, and HSG258, paragraphs 318 and 319.

LEV log book

# Maintenance, replacements and repairs

Record every service, part replaced and repair, planned or not, and who did the work.

| Date | Part of the system | Work done | Planned or unplanned | Parts replaced | Done by (name and company) | Signature |
| --- | --- | --- | --- | --- | --- | --- |

Source: HSE guidance HSG258, paragraphs 318 and 319.

LEV log book

# Faults and actions

Record each fault found by a check or a test, what will put it right, and when it was done. COSHH requires a record of the repairs made as a result of each thorough examination and test.

| Date found | Fault | Found by | Action needed | Who will do it | Date done | Signed off by |
| --- | --- | --- | --- | --- | --- | --- |

Source: COSHH Regulations 2002, regulation 9(4).

LEV log book

# Thorough examination and test record

COSHH requires a thorough examination and test at least once every 14 months, or more often for the processes in its Schedule 4. Keep each full report, and the record of repairs that followed it, for at least five years, with a copy at the workplace.

| Test date | Examiner and company | Report reference | Hoods passed | Hoods failed | Critical faults | Repairs completed on | Next test due |
| --- | --- | --- | --- | --- | --- | --- | --- |

Source: COSHH Regulations 2002, regulation 9(2) and (4), and HSG258, paragraph 350.

LEV log book

# Before the examiner visits

HSE guidance lists what the examiner should use at each thorough examination and test, where it exists. Tick each one when it is ready.

Test booked for: ________________________________________________

Examiner and company: ________________________________________________

Contact on the day: ________________________________________________

☐ Commissioning report

☐ User manual

☐ This log book

☐ Last thorough examination and test report

☐ Air sampling records, and notes on how operators use the system

☐ Confirmation that the system, the layout and the process have not changed since the last test, or a list of what changed

Source: HSE guidance HSG258, paragraph 334.